Nonconformance Report NCR-2026-014

Cascade Precision Components - Quality Records

Part: CPC-84122 rev A - HV-Series Poppet, Hardened (SAFETY CRITICAL)

Lot: LOT-2026-0117    Supplier: Sound Heat Treat    Qty: 300

Opened: 2026-06-16 by Y. Mori    Severity: CRITICAL    Defect: MATERIAL

Description:

Certification review found the heat-treat charge processed to AMS 2759/3 class per a superseded revision. Core hardness verified in-spec on a 12-piece sample, but the material condition does not match the drawing callout for this safety-critical part.

Disposition: USE_AS_IS

MRB approval: NOT RECORDED - engineering rationale attached but the Material Review Board has not signed. AS9100 8.7 requires MRB sign-off for USE_AS_IS on MAJOR/CRITICAL severity. Flagged by internal audit Q2-2026; status remains Dispositioned, not Closed.

Linked records: lot LOT-2026-0117 (also missing CoC), PO-30000108.

All Cascade Precision Components data is fictional and generated for training.