8D Report - Customer Escape, Plating Adhesion

Cascade Precision Components - Corrective Action 8D-2026-03

D1 Team: Y. Mori (lead), F. Okonkwo, P. Novak, S. Grant

D2 Problem: Tidewater Marine Systems reported plating flaking on QC fittings CPC-61180 from shipment SH-70000304 (500 pcs, lot LOT-2026-0107, supplier Pacific Alloys Finishing).

D3 Containment: lot RECALLED 2026-06-20; customer stock returned (shipment status Returned); remaining stock quarantined.

D4 Root cause: supplier bath chemistry out of control; no rack-to-lot traceability at supplier (see supplier audit 2026-05-27).

D5/D6 Corrective actions: supplier SUSPENDED on the ASL (2026-05-30); NCR-2026-016 dispositioned RTV; alternate plater qualification started with Sound Heat Treat.

D7 Prevent recurrence: incoming plating adhesion test added for all plated fittings; ERP block for POs to suspended suppliers REQUESTED (gap demonstrated by PO-30000112).

D8 Closure: open - pending customer credit and requalification.

All Cascade Precision Components data is fictional and generated for training.